Mastermind Report

Course: F28HS Hardware-Software Interface

Group:44

Group member 1: (H00409464) Ahlam Rafati

Group member 2: (H00402356) Syed Taha Rahman

1- Short Problem Specification

This coursework aims to develop a systems-level application that implements the board game MasterMind. This application is specifically designed to operate on a Raspberry Pi and to interact with external hardware components, which include two LEDs, a button, and an LCD. The system, functioning as the codekeeper, generates a random secret sequence and provides feedback to the codebreaker through LED blinks and LCD output, while the codebreaker inputs guesses through the button. The challenges associated with this project include low-level hardware control, precise timing for user inputs, and the integration of C programming with ARM Assembler to ensure optimal performance operations.

2-The hardware specification and wiring

The chosen hardware platform for the project consists of a Raspberry Pi (models 3), which is compatibility with the 32-bit Raspberry Pi OS necessary for the proper functioning of ARM Assembler. The devices that are connected include.

- LEDs:

  - Green LED: Connected to GPIO pin 26, used for data feedback (e.g., echoing button input or indicating exact/approximate matches).

  - Red LED: Connected to GPIO pin 5, used to signal control information.

- Button:

  - Connected to GPIO pin 19, the button is used to record user inputs. Each number is input as a series of presses, with a fixed time-out separating consecutive numbers.

- LCD Display:

  - Wired through GPIO pins 23, 10, 27, and 22. The LCD also receives power via the 5V line and is connected using a breadboard setup.

3-Code Structure

The software is organized into distinct modules to separate hardware control from game logic:

Initialization Module

- GPIO Setup: Configures the Raspberry Pi’s GPIO pins for input (button) and output (LEDs, LCD).

- LCD Initialization: Sets up the LCD display.

- Timer Setup: Initializes timers (either in C or ARM Assembler) required for the fixed time-out between button presses.

Game Logic Module (C)

- Secret Sequence Generation: Randomly generates a secret sequence that the codebreaker must guess.

- Input Handling: Reads a sequence of numbers entered via the button, using a fixed time-out to delimit each number.

- Match Calculation: Determines the number of exact matches (right colour and position) and approximate matches (right colour, wrong position) between the guess and the secret sequence.

- Feedback and Control: Directs the LED blinking patterns and outputs to the LCD for user feedback.

Hardware Control Functions (C and ARM Assembler)

- Direct Hardware Access Functions:

C Functions:

- pinMode: Configures GPIO pins.

- digitalWrite: Writes a high or low value to a GPIO pin.

- readButton: Reads the current state of the button.

- waitForButton: Implements a delay waiting for button input.

ARM Assembler Functions:

writeLED: Low-level routine for controlling LED output.

Matching Function:

Name: (e.g., mm_matches)

Interface Discussion:

Inputs: Two sequences (the secret sequence and the user guess), along with the length (N) and the number of colours (C).

Output: Two integers representing the number of exact matches and approximate matches.

Implementation: The matching function is implemented in ARM Assembler to optimize performance and ensure real-time responsiveness. It adheres to the ARM subroutine calling conventions (AAPCS).

Command-Line and Debug Modes

Command-Line Options:

- -d for debug mode (prints secret and input sequences).

- -s <secret sequence> to specify a fixed secret for testing.

- -u <seq1> <seq2> to run unit tests on the matching function.

4- A discussion of performance-relevant design decisions

Key design decisions include:

- ARM Assembler for Time-Critical Operations:
Implementing functions such as the matching algorithm and low-level GPIO control in ARM Assembler reduces inactivity and ensures accurate timing. This is especially important when debouncing button inputs and performing precise LED blink sequences.

- Fixed Time-Out Implementation:
A fixed time-out between button presses allows the system to distinguish separate numeric inputs. By using either hardware timers, the system avoids misinterpretation of input sequences, ensuring that users experience responsive and accurate feedback.

- Separation of Hardware and Game Logic:
The design minimizes potential overhead and interference in time-sensitive operations by isolating hardware control functions from game logic.

- Use of the 32-bit Raspberry Pi OS:
Ensuring the use of the 32-bit version of Raspberry Pi OS guarantees compatibility with ARM, therefore optimizing the performance of the low-level hardware interfaces.

5. Example Output from a Complete Game

- At Startup (Greeting):

For the surname “Rafati”, the system blinks: Red, Green, Red, Green, Red, Green.

- Game Rounds:

Secret Sequence is Generated. (In debug mode, the secret sequence is shown, e.g., 1 2 2.)

- User Input Sequence:

- First number: Button pressed 3 times → LED feedback: Red blink, then Green blinks 3 times.

- Second number: Button pressed 1 time → LED feedback: Red blink, then Green blink once.

- Third number: Button pressed 2 times → LED feedback: Red blink, then Green blinks 2 times.

- End of Input:

- Red LED blinks twice to signal the end of the guess.

- Feedback on Guess:

- Green LED blinks to indicate the number of exact matches (e.g., 1 blink for 1 exact match).

- Red LED blinks once as a separator.

Green LED blinks to indicate the number of approximate matches (e.g., 1 blink for 1 approximate match).

- Round Transition:

- Red LED blinks three times to signal the start of a new round.

- Successful Guess:

1.  If the codebreaker guesses correctly, the system displays “SUCCESS” on the LCD along with the number of attempts, and the green LED blinks three times while the red LED stays on. Else the system displays “Game Over” message on the screen.

6- Summary and Reflection

We developed a MasterMind game application using C and ARM Assembler for effective, low-level hardware control. Key achievements include: • Accurate Hardware Interfacing: The system reads user input when the button is pressed, controls LEDs for feedback, and displays the output on LCD. • proper Use of Timers and Assembler: By using fixed time and implementing important steps in ARM Assembler, the application meets the requirements. • Modular Code Structure: The separation between hardware control and game logic made the code maintainable and easier to test. • Learning Outcomes: Through this project, we understood systems programming, improved our skills in low-level hardware, and gained practical experience in understanding more complicated code. The project achieved the required goals, but it could be improved by handling more errors and adding more features to the LCD. Overall, this coursework gave us a useful knowledge of how to link the software with hardware.